Date of Assessment: October 13, 2026

Location: Bolo Town, Nimba County

Sample Matrix: Water

Analysis Start Date: October 15, 2026

Table 1: Surface Water Quality Sample Information

| Sample Location | Sample ID | GPS Coordinates |  | Sampling Time | Description |
| --- | --- | --- | --- | --- | --- |
|  |  | Latitude | Longitude |  |  |
| Wenee Downstream | WE-JC-SW-001 | 0542501 | 0834879 | 2:08 PM | — |
| Wenee mid-point | WE-JC-SW-002 | 0542460 | 0834664 | 2:21 PM | — |
| Wenee Upstream (confluence with Saymenee) | WE-SW-JC-003 | 0542353 | 0834524 | 2:36 PM | — |
| Saymenee mid-point | SM-SW-JC-001 | 0542304 | 0834561 | 2:45 PM | — |
| Alphonso farm | AF-SW-JC-001 | 7.547899 | -8.616177 | 2:21 PM | — |
| Neneyee Upstream (northern sediment control station) | NEN-SW-JC-001 | 7.537928 | -8.614119 | 4:24 PM | — |
| Wehyee Downstream | W-SW-VA-001 | 7.56771 | -8.60949 | 2:58 PM | — |
| Wehyee Mid-point | W-SW-VA-002 | 7.56695 | -8.61227 | 3:16 PM | — |
| Wehyee Upstream | W-SW-VA-003 | 7.56724 | -8.6128 | 3:21 PM | — |
| Pompleh Downstream | POM-SW-VA-001 | 7.56793 | -8.6094 | 3:03 PM | — |
| Pompleh Mid-point | POM-SW-VA-002 | 7.56822 | -8.60947 | 3:07 PM | — |
| Pompleh Downstream | POM-SW-VA-003 | 7.56882 | -8.60976 | 3:35 PM | — |
| Yeewa Downstream | YEEWA-DS | 542444 | 834881 | 2:09 PM | Downstream surface water sample collected from the Yeewa main stream, a natural water body, below the inflow of the Yeewa Korweny 1 and 2 tributaries. Iron ore sediment was observed along the waterway and turbidity was visibly higher than at Yeewa Korweny 1. |
| Yeewa Mid-Stream | YEEWA-MS | 542314 | 834915 | 2:16 PM | Mid-stream surface water sample collected along the Yeewa main stream to assess spatial sediment migration. Reddish iron ore sediment deposits were observed along the channel, with elevated turbidity. |
| Yeewa Up-Stream | YEEWA-US | 542066 | 835147 | 2:32 PM | Upstream surface water sample collected on the Yeewa main stream above the tributary junctions. Iron ore sediment was still present along the waterway at this point, indicating sediment loading from further upstream in the Mount Yewlliton drainage. |
| Yeewa Korweny Patel Downstream | YWK2-ES-SW-001 | 0541616 | 0835024 | 3:15 PM | Downstream surface water sample collected from Yeewa Korweny 2 (Patel), a tributary of the Yeewa main stream, just above its junction with the main watercourse. Substantial sediment deposits were observed and turbidity was higher than at Yeewa Korweny 1. |
| Yeewa Korweny Patel Midstream | YWK2-ES-SW-002 | 0541600 | 0834909 | 3:18 PM | Midstream surface water sample collected along Yeewa Korweny 2 (Patel) within a deep, densely forested valley. A geo-fencing sign reading “KEEP AWAY – 500 Meters Safety Buffer Zone” was observed on this tributary, in proximity to ArcelorMittal’s haulage route. |
| Yeewa Korweny Patel Upstream | YWK2-ES-SW-003 | 0541550 | 0834786 | 3:21 PM | Upstream surface water sample collected on Yeewa Korweny 2 (Patel) towards its source in the Mount Yewlliton area, in proximity to mining activities in the Mount Yewlliton and Mount Gangro areas. Sediment deposits were observed in the channel. |
| Yeewa korweny Dor Downstream | YWK1-ES-SW-001 | 0541697 | 0835051 | 3:35 PM | Downstream surface water sample collected from Yeewa Korweny 1 (Dor), the tributary on the opposite side of the Yeewa main stream from Yeewa Korweny 2, near its junction with the main watercourse. Sediment deposits were present but turbidity was visibly lower than at Yeewa and Yeewa Korweny 2. |
| Yeewa Korweny Dor Midstream | YWK1-ES-SW-002 | 0541644 | 0835167 | 3:44 PM | Midstream surface water sample collected along Yeewa Korweny 1 (Dor) as it flows through a deep valley under dense forest cover. Sediment deposits were observed, with lower turbidity than the Yeewa Korweny 2 branch. |
| Yeewa korweny Dor Midstream | YWK1-ES-SW-003 | 0541607 | 0835271 | 3:59 PM | Upper surface water sample collected on Yeewa Korweny 1 (Dor) towards its source in the Mount Yewlliton area, located in proximity to ArcelorMittal’s haulage route and the Mount Yewlliton / Mount Gangro mining areas. Natural water body with sediment deposits in the channel. |

Note: Descriptions for the ES group sampling points (shaded) are derived from “ES Group – Physical Observations at Following Sampling Points”. YWK2 = Yeewa Korweny 2 (Patel); YWK1 = Yeewa Korweny 1 (Dor). Descriptions for the JC and VA group points are to be supplied by those groups.

PHYSICAL OBSERVATIONS AT SAMPLING POINTS – ES GROUP (YEEWA, YEEWA KORWENY 1 & 2)

Sampling was followed by a physical inspection of the Yeewa main stream and its Yeewa Korweny 1 (Dor) and Yeewa Korweny 2 (Patel) tributaries. The following preliminary observations were recorded by the ES group:

An agricultural farm was observed to have been inundated and covered by sediments originating from the mining area. This may affect the overall productivity of the farmland.

The presence of iron ore sediment was observed along the waterway during the sampling.

The turbidity levels at Yeewa and Yeewa Korweny 2 were higher than those at Yeewa Korweny 1.

A geo-fencing sign was observed at Yeewa Korweny 2 bearing the inscription: “KEEP AWAY – 500 Meters Safety Buffer Zone”.

All the sampling points were located within natural water bodies.

Yeewa Korweny 1 and Yeewa Korweny 2 are tributary branches of the Yeewa main stream, situated on opposite sides of the main watercourse. Both originate in the Mount Yewlliton area and flow through deep valleys characterised by dense forest cover. The tributaries were also observed to contain substantial sediment deposits. Their locations are in proximity to ArcelorMittal’s haulage route and to mining activities in the Mount Yewlliton and Mount Gangro areas. The proximity of mining and haulage activities, together with the observed sedimentation, warrants further investigation into possible sediment transport pathways. Nevertheless, the field observations alone do not establish that ArcelorMittal’s activities caused the sedimentation or any chemical pollution.

Table 2: Surface Water Quality Result

| Sample Code | Sample Parameter | LWQS Class III | Instrumentation |
| --- | --- | --- | --- |
|  | Total Suspended Solid (TSS) |  |  |
| YEEWA-DS | 532 | ≤50.0 | Colorimeter |
| YEEWA-MS | 523 |  |  |
| YEEWA-US | 628 |  |  |
| YWK2-ES-SW-001 | 1858 |  |  |
| YWK2-ES-SW-002 | 2,400 |  |  |
| YWK2-ES-SW-003 | 769 |  |  |
| YWK1-ES-SW-001 | 259 |  |  |
| YWK1-ES-SW-002 | 246 |  |  |
| YWK1-ES-SW-003 | 188 |  |  |
| W-SW-VA-001 | 24 |  |  |
| W-SW-VA-002 | 24 |  |  |
| W-SW-VA-003 | 20 |  |  |
| POM-SW-VA-001 | 11 |  |  |
| POM-SW-VA-002 | 24 |  |  |
| POM-SW-VA-003 | 164 |  |  |
| WE-JC-SW-001 | 102 |  |  |
| WE-JC-SW-002 | 118 |  |  |
| WE-SW-JC-003 | 214 |  |  |
| SM-SW-JC-001 | 84 |  |  |
| AF-SW-JC-00 | 1067 |  |  |
| NEN-SW-JC-001 | 92 |  |  |